Christ the Redeemer Catholic Schools

Hearing Conservation

Code of Practice

Table of Contents:

Overview……………………………………………………………………………………..2

Legislative Requirements………………………………………………………….….3

Guidelines for Meeting Legislative Requirements…………………………5

Training………………………………………………………………………………..………7

Implementation……………………………………………………………………….…..8

Appendix 1……………………………………………………………………………….…..9

Overview

Occupational noise is a potential on-the-job health hazard. The hazard is not always obvious at

the time, but years down the road employees exposed to excessive noise levels over extended

periods of time may develop hearing loss. Noise-induced hearing loss may be permanent

and irreversible.

Employees may be exposed to excessive noise from a variety of sources including some

that are non-work related. Some work-related examples might be:

• Musical instruments.

• Pneumatic tools - drills, saws.

• Power tools -hand drills, saws, lathes.

Legislative Requirements

Under provincial OHS legislation, the employer must take steps to protect the hearing of

employees who are or may be exposed to excessive noise levels.

Hearing conservation is regulated under:

Alberta Occupational Health & Safety Act, Regulation and Code

- Noise Exposure, Part 16, Section 216.

Noise Exposure Regulation

The Noise Exposure Regulation establishes occupational exposure limits (OEL) for

noise. The OEL is the maximum sound level to which the average employee may be

exposed for a specified time without adverse effects to their hearing. OELs are indicated

in the tables in Appendix I of this section.

The employer must:

• Take all reasonable steps to ensure that no employee is exposed to noise in excess

of the OEL by instituting engineering controls, work practice or administrative

controls.

• If these measures fail in keeping noise exposure under the OELs, supply specified

protective equipment to the employee; post signs to indicate that a noise hazard exists,

and the protective equipment required; and ensure that the exposed employee is informed

of the hazards and the purpose and limitations of protective equipment.

• Identify those work situations where a noise hazard exists and ensure that a noise

exposed employee undergoes approved audiometric testing regularly.

A noise-exposed employee is one who is exposed to noise above the OELs. A noise-exposed

employee must wear the hearing protection provided and submit to audiometric testing

when the District identifies their work situation as being noise exposed.

To meet legislative requirements, the school or workplace should develop a

hearing conservation program. The goal of an occupational hearing conservation program is to prevent noise-induced hearing loss resulting from exposures to noise at work.

Hearing Conservation Compliance

Hearing conservation is judged from a prevention standard.

The questions a government inspector would ask are:

• Employees are warned that their area of teaching has excessive noise levels and may   be at risk.

• Do employees know they are expected to wear hearing protection when exposed to

noise above the prescribed Alberta Occupational Exposure Limits for noise?

• Do employees know hearing protective equipment must meet the CSA Standard?

• Do employees who teach Band and CTS Construction, know they must undergo audiometric testing as being in a noise exposed occupation?

If the answers to the above questions are YES, then there is compliance.

At Christ the Redeemer Catholic Schools, Band and CTS Construction teachers are classified as Noise Exposed Workers as they:

• May be subjected to more noise than the occupational exposure limit (OEL).

• Where hazardous noise is identified, and where practical, eliminated through engineering controls, e.g., sound barriers and noise dampers, work practice or administrative controls.

• Audiometric testing is conducted by an audiometric technician on employees at regular intervals or prescribed by attending Audiologist; or, if the employee is exposed to noise.  A copy of these results will be provided to the employee.

• Employees are educated about the hazards of high noise levels and correct use of control measures and hearing protection.

• A supply of approved earmuffs or other protective devices (musician earplugs) are available to those employees exposed to continuous noise.

Guidelines for Meeting Legislative Requirements

Noise Exposure Assessment

• If there are concerns, contact the Health and Safety Officer to arrange for a

noise assessment who will arrange for appropriate testing.

• A reassessment should be considered if a change in process or work environment occurs.

The assessment shall include:

- A general statement of scope of work carried out and an outline of activities or

processes measured.

- The date, time, location, and duration of noise samples.

- Type and Calibration data on each piece of instrumentation used.

-  Sound level readings measured

- Suggestions of possible control strategies and needs specific to this workplace.

Audiometric Testing

• Those employees identified by the District as noise-exposed shall have regular

audiometric tests (band teachers).  For the implementation of this Hearing Conservation Program, tests should be completed within the first 3 months of employment.  The next test should be conducted 1 year later; and every 2 years after, as long as they are employed with CTR.

• A new employee shall be tested prior to working in a District identified noise exposed

Area, to establish a baseline audiogram.  Subsequently, within 12 months of the baseline audiogram and every two years, until leaving the District or changing work locations.

• The audiometric testing shall be conducted by a certified audiometric technician

who has completed an approved course recognized by Alberta Human Resources

and Employment.

• Audiometric testing coverage is approved under the Alberta School Employee Benefit Plan coverage.

• If any of the audiograms of employees are categorized as “abnormal” or “abnormal

shift”, the employee will be informed and be referred to a physician or an audiologist specialist.

• Confidential employee medical records must be kept by the attending Audiologist to enable tracking employee’s hearing performance and in evaluating a hearing loss claim, if applicable.

• The Health and Safety Officer will collaborate on noise-exposed employees if requested by the Alberta Human Resources and Employment.

Hazard Warning

• Christ the Redeemer Catholic Schools recognizes that band and CTS construction classrooms are noise exposed environments.

• Christ the Redeemer Catholic Schools recognizes that some facility personnel may be periodically exposed to excessive noise and shall wear appropriate ear protection.

Hearing Protection

• All band and CTS construction teachers, as well as facility personnel shall, be supplied with hearing protector that is CSA approved and receive instruction from the supplier, as to care, proper fit, and usage (musician earplugs, earmuffs, or disposable earplugs).

• Employees are required to always wear hearing protection when working in

areas that have noise.

• Employees shall not modify the hearing protector in a way that would reduce its

noise attenuation characteristics.

The standards in this section are minimum standards (see Appendix I). The focus of

workplace accident and injury prevention efforts should be to exceed minimum standards.  Noise assessment conducted in the first 6 months of 2008 revealed that band teachers are within 6 dBA of the provincial guidelines, and have instituted a hearing conservation program for band and CTS construction teachers.

Training Requirements

Employees working in a noise-exposed area of the workplace, shall review this Hearing Conservation Code of Practice, as identified on the Job Hazard Assessment to understand noise, its effects, and how to prevent hearing loss.

Retraining should occur when:

• New work processes or equipment are introduced into the school or workplace.

• New hearing protection equipment is purchased.

• There are changes in legislation.

Implementation

Getting Started

- CTS Construction and Band teachers to review the Hearing Conservation Program as outline in the Job Hazard Assessment

2. 	Develop plan for reducing noise at the source, by identifying situations where

improvement can be made by:

• Substituting quieter equipment or machinery for noisy equipment or machinery

or quieter processes when and where feasible.

• Modifying equipment by running equipment more slowly, improving lubrication,

balancing rotating parts or reducing vibration.

• Isolating noise by using sound barriers, partitions, adding sound absorbent

material around noisy equipment, or enclosing a noisy machine in a sound

proof room.

• Maintenance/inspection of equipment.

- Develop a system for preventive maintenance of hearing protection equipment.

4. 	Employees follow hearing conservation guidelines on excessive noise levels.

Ongoing Activities

Principals and non-school based department heads shall:

1. Monitor employees to see if hearing protection equipment is being worn as required.

2. Monitor the preventive maintenance program to ensure high standards for hearing protection.

3. Annual monitoring of the hearing conversation program.

Appendix I

Occupational Exposure Limits for Noise

| Exposure Level (dBA) | Exposure Duration |
| --- | --- |
| 82 | 16 hours |
| 83 | 12 hours and 41 minutes |
| 84 | 10 hours and 4 minutes |
| 85 | 8 hours |
| 88 | 4 hours |
| 91 | 2 hours |
| 94 | 1 hour |
| 97 | 30 minutes |
| 100 | 15 minutes |
| 103 | 8 minutes |
| 106 | 4 minutes |
| 109 | 2 minutes |
| 112 | 56 seconds |
| 115 or greater | 0 |

NOTE:  Exposure levels and exposure duration to be prorated if not specified.

Selection of Hearing Protection Devices

| Maximum Equivalent Noise Level (bBA Lex) | CSA Class of Hearing Protection | CSA Grade of Hearing Protection |
| --- | --- | --- |
| <90 | C,B or A | 1,2,3 or 4 |
| <95 | B or A | 2,3 or 4 |
| <100 | A | 3 or 4 |
| <105 | A | 4 |
| <110 | A earplug + A or B earmuff | 3 or 4 earplug + 2,3 or 4 earmuff |
| >110 | A earplug + A or B earmuff and limited exposure time to keep sound reaching the worker’s eardrum below 85 dBA Lex | 3 or 4 earplug + 2,3 or 4 earmuff and limited exposure time to keep sound reaching the worker’s ear drum below 85 dBA Lex |

- dBA means a measure of sound level in decibels using a reference sound pressure of 20 micropascals when measured on the A-weighting network of a sound level meter.

- “Lex” means the level of a worker’s total exposure to noise in dBA averaged over the entire workday and adjusted to an equivalent eight hour exposure.

Permissible Background Noise Conditions During Audiometric Testing

| Octave Band Centre Frequency (Hz) | Maximum Level (dB) |
| --- | --- |
| 500 | 22 |
| 1000 | 30 |
| 2000 | 35 |
| 4000 | 42 |
| 8000 | 45 |

Noise Level – Examples

| Level (decibels) | Example | Maximum Hours/Day of Exposure |
| --- | --- | --- |
| 65 | Normal Speaking | No Limit |
| 75 | Average Factory | No Limit |
| 86 | Lawn Mower | 8 hours |
| 92 | Compressor | 4 hours |
| 95 | Bandsaw | 2 hours |
| 99 | Siren | 2 hours |
| 100 | Chain Saw | 1 hour |
| 105 | Router | 1 hour |
| 119 | Average Car Horn | 2 minutes |

- To reduce or eliminate the risk of injury owing to continuous noise levels, the appropriate precautions should be taken, by using hearing protection and/or limiting exposure time.  The most common types of hearing protection are earplugs and earmuffs.